PROCESS  •  PROPOSAL  •  EPC ENGINEERING

Topic-Wise Learning &

Interview Guide

For a cement tendering / proposal engineer with ~4 years of experience

Basics  →  Advanced  →  Interview Questions

20 Topics  |  16-Week Study Plan  |  Mock Project  |  Rapid-Fire Numbers

Career track:  Tendering Engineer → Process / Proposal Engineer → Process Design / EPC Engineer

Contents

# Process / Proposal / EPC Engineer — Topic-Wise Learning & Interview Guide

Target profile: ~4 years cement tendering/proposal experience → Process & Proposal / EPC Process Engineer

Structure per topic: Basics → Advanced → Interview Questions (with model answers at 4-yr level)

## 0. How to read this guide

| Priority | Topics | Time |
| --- | --- | --- |
| Tier 1 (must have) | Mass balance, Energy balance, Process calcs, PFD, P&ID, Cement process, Equipment fundamentals, Excel, Tendering | Weeks 1–8 |
| Tier 2 (very useful) | AutoCAD, HAZOP, Datasheets, TBE, CAPEX/OPEX, Utilities, Control philosophy, Cause & effect | Weeks 8–16 |
| Tier 3 (after job) | Aspen/HYSYS, process control, Power BI, Python, APC | Month 5+ |

Study rhythm (2–3 h/day): 60% concept + 30% hand calculation in Excel + 10% interview rehearsal.

Golden rule: every topic must end with one worked numerical example you can reproduce on a whiteboard.

# TOPIC 1 — MASS BALANCE

### Basics

- Conservation of mass: Input = Output + Accumulation ± Generation/Consumption

- Steady state → accumulation = 0. Batch/startup → accumulation ≠ 0.

- Basis selection: 1 h, 1 t clinker, 100 kg feed. Always state the basis.

- Streams: total mass balance + component balance. Degrees of freedom = unknowns − independent equations.

- Terms: tie component, recycle, purge, bypass, split fraction, conversion, yield, selectivity.

- Units discipline: t/h, kg/s, Nm³/h vs Am³/h, % by mass vs % by volume, dry vs wet basis.

### Advanced

- Recycle loops: solve with overall balance first, then unit balance — avoids simultaneous equations. In cement: cyclone dust return, separator rejects to VRM, kiln dust cycle.

- Tie-component method: use ash/inert or an unreacted species to link streams (used in raw meal ↔ clinker with LOI as the "tie").

- Clinker factor: raw meal to clinker ≈ 1.55–1.65 kg raw meal/kg clinker (LOI ~35% + dust losses).

- Volatile cycles: alkali/sulphur/chloride circulation, enrichment factor, bypass sizing (typically 3–10% kiln gas bypass for chloride control).

- False air ingress: quantified via O₂/CO₂ balance across preheater — a classic audit calculation.

- Gas balance: combustion stoichiometry, excess air, Nm³ of gas per kcal, dust loading in g/Nm³.

### Worked example to memorise

Produce 5000 TPD clinker. Raw meal factor 1.58 → raw meal = 7900 TPD. Raw mix moisture 4% → wet feed = 7900/0.96 ≈ 8229 TPD. Coal at 3.2 GJ/t clinker with GCV 22 MJ/kg → 727 t/d coal.

### Interview questions

- What is the raw meal to clinker factor and why is it ~1.55–1.6?

Because of LOI (CO₂ from calcination ≈ 35% of raw meal) plus dust losses and bypass dust removal.

- How do you check whether a plant mass balance is correct? Closure error < 2–3%, cross-check with a tie component, verify with independent measurement (belt weigher, gas analyser).

- A recycle stream increases in a grinding circuit — what changes? Circulating load rises, mill throughput per unit fineness changes, separator efficiency and specific power alter; check for coating/blinding or worn separator.

- Difference between wet basis and dry basis, and where does it bite you in a tender? Guarantees are usually on dry basis; quoting fan volume on the wrong basis under-sizes the fan.

- How do you estimate false air in a preheater string? Compare O₂ at kiln inlet vs preheater exit; use the nitrogen balance to compute added air.

- Degrees of freedom — how do you decide a problem is solvable? Count unknown stream variables vs independent balances + specified relations.

# TOPIC 2 — ENERGY BALANCE & THERMODYNAMICS

### Basics

- Energy in = Energy out + losses. Sensible heat Q = m·Cp·ΔT, latent heat, heat of reaction.

- Enthalpy reference temperature (usually 20 °C or 0 °C) — must be stated.

- Fuel: GCV vs NCV, ash, moisture, proximate/ultimate analysis.

- First law, second law, entropy, efficiency vs effectiveness.

- Cement KPIs: specific heat consumption (kcal/kg clinker), specific power (kWh/t cement).

### Advanced

- Heat of clinkerisation ≈ 420–450 kcal/kg clinker (theoretical). Modern 5-/6-stage preheater–precalciner kiln ≈ 690–730 kcal/kg; long dry kiln 900+; wet process 1300+.

- Kiln heat balance items: fuel input, sensible heat of feed/air/fuel vs clinker exit heat, cooler exhaust loss, preheater exit gas loss (biggest, ~20%), radiation/convection from shell, moisture evaporation, dust loss, bypass loss.

- Cooler efficiency: recuperation efficiency 70–75% modern cross-bar coolers; secondary air 950–1100 °C; tertiary air duct.

- Waste Heat Recovery (WHR): AQC + PH boilers, typical 25–35 kWh/t clinker; ORC vs steam Rankine; economics for tenders.

- Alternative fuels (AFR): TSR%, effect of low NCV/high moisture/chlorine on kiln stability and bypass.

- Specific power benchmarks: raw grinding VRM 14–18, cement grinding VRM 24–30 / ball mill 32–40, kiln section 22–26, total 90–110 kWh/t cement.

### Interview questions

- Where does most heat go in a cement kiln? Clinkerisation reaction (~50%), preheater exhaust gas, cooler vent air, clinker sensible heat, shell radiation.

- How would you reduce heat consumption from 760 to 700 kcal/kg? Add a preheater stage, reduce false air, improve cooler recuperation, optimise burner/flame, insulate, control excess air, stabilise kiln feed.

- GCV vs NCV — which do you use for kiln heat balance and why? NCV, since water vapour leaves the stack uncondensed.

- What is the theoretical vs actual heat of clinkerisation and why the gap? ~420 vs ~700 kcal/kg due to unrecoverable exhaust/radiation/cooler losses.

- How does a WHR system affect your proposal economics? Reduces grid power by ~25–30%, payback 3–5 years; but reduces gas temperature available for raw mill drying — must be balanced.

- How do you handle high-moisture raw material in the heat balance? Extra drying duty; check if PH exit gas heat is sufficient or an auxiliary hot gas generator is required.

# TOPIC 3 — FLUID MECHANICS & HYDRAULICS

### Basics

- Continuity Q = A·V; Bernoulli; static/dynamic/total pressure.

- Reynolds number, laminar/turbulent, friction factor (Moody), Darcy–Weisbach hf = f·L·V²/(2gD).

- Minor losses (K-values), equivalent length.

- Typical velocities: water pipe 1–3 m/s, gas duct 15–20 m/s, dust-laden duct ≥18 m/s (to avoid settling), compressed air 15–25 m/s.

### Advanced

- NPSH available vs required, cavitation, vapour pressure margin (min 1 m or 1.3× NPSHr).

- Pump curve vs system curve, operating point, parallel/series operation, VFD affinity laws:

Q ∝ N, H ∝ N², P ∝ N³.

- Two-phase/pneumatic conveying: dilute vs dense phase, saltation velocity, air-to-material ratio.

- Duct system draft balance, damper vs VFD control, fan surge, stack effect.

- Slurry/abrasive handling, pipe wear, erosion velocity limits.

### Interview questions

- Pump is cavitating — diagnose. Low suction level, high suction line loss, clogged strainer, high liquid temperature, air ingress; fix by raising NPSHa or reducing NPSHr.

- Why is VFD better than damper control? Damper wastes energy as throttling loss; VFD reduces power by cube law — a 20% flow cut saves ~50% power.

- How do you size a pipe? Fix velocity range → compute diameter → check pressure drop and economic diameter → round to standard NB → verify NPSH/pump head.

- Series vs parallel pumps? Series adds head, parallel adds flow (with flatter net gain due to system curve).

- Why minimum velocity in a dust-laden duct? Below saltation velocity dust settles, causing blockage and duct collapse risk.

# TOPIC 4 — HEAT TRANSFER

### Basics

- Conduction (Fourier), convection (Newton's law of cooling), radiation (Stefan–Boltzmann).

- Q = U·A·LMTD·F, resistances in series, fouling factor.

- Counter-current vs co-current; LMTD correction factor F for shell & tube.

### Advanced

- Heat exchanger types: shell & tube (TEMA types), plate, air-cooled; when to use each.

- Effectiveness–NTU method for gas-gas exchangers.

- Cement-specific heat transfer: cyclone gas–solid heat exchange, cooler grate recuperation, kiln shell radiation losses, refractory & insulation design (hot face/cold face temperature calc).

- Insulation economics: heat loss vs insulation thickness, critical radius.

### Interview questions

- Why counter-current is preferred? Higher LMTD, greater duty for same area, can achieve temperature cross.

- Fouling factor — how does it affect design? Increases required area (over-surface 10–25%), lowers U; matters for the guarantee margin in tenders.

- How would you calculate kiln shell heat loss? Shell temperature scan + area + combined radiation/convection coefficient (typ. 15–25 W/m²K) → kcal/kg clinker.

- A preheater cyclone shows low efficiency — heat transfer or separation issue? Check both: pressure drop, dip tube condition, gas velocity, dust cycle.

# TOPIC 5 — UNIT OPERATIONS (PROCESS FUNDAMENTALS)

### Basics

Crushing & grinding, screening & classification, mixing/blending, drying, separation (cyclones, filters), pneumatic/mechanical conveying, storage & flow of solids, combustion.

### Advanced

- Comminution laws: Rittinger, Kick, Bond — W = 10·Wi·(1/√P80 − 1/√F80). Bond Work Index for limestone ~10–13 kWh/t.

- Grindability: Hardgrove index for coal, Blaine fineness (cement 300–400 m²/kg OPC, 400–500 PPC), residue on 45 µm.

- Particle size distribution, RRSB slope n, effect of separator efficiency (Tromp curve, bypass, cut size d50).

- Drying: drying curve, critical moisture, gas quantity required per kg water.

- Solids handling: angle of repose, arching/rat-holing, mass flow vs funnel flow silos.

### Interview questions

- VRM vs ball mill — compare. VRM: 30–40% lower power, integrated drying/classification, compact, sharper PSD but narrower (affects water demand); ball mill: robust, higher power, better for high-Blaine/slag, higher maintenance downtime tolerance.

- What is a Tromp curve? Separator efficiency vs particle size; shows bypass and d50 cut size and sharpness.

- How do you decide mill capacity in a proposal? Required output at guaranteed fineness × safety margin, from Bond/vendor grindability tests + moisture and hardness data.

- Cement Blaine increased but strength did not — why? PSD spread, gypsum optimisation, clinker mineralogy (C3S), false set, mill temperature.

# TOPIC 6 — CEMENT PROCESS (YOUR DOMAIN EDGE)

### Basics — flow chain

Quarry → Crusher → Stacker/Reclaimer (pre-blending) → Raw grinding (VRM/ball) → Blending silo → Preheater (4–6 stage) → Calciner → Rotary kiln → Clinker cooler → Clinker silo → Cement grinding (+gypsum, fly ash/slag) → Cement silo → Packing/Dispatch.

### Key chemistry

- Main phases: C3S (alite), C2S (belite), C3A, C4AF.

- Moduli: LSF (92–98), SM/SR (2.3–2.7), AM/AR (1.3–1.6).

- Bogue equations; free lime target 0.8–1.5%; litre weight 1150–1350 g/l.

- Calcination CaCO₃ → CaO + CO₂ at ~900 °C; clinkerisation 1350–1450 °C; burning zone flame ~1900–2000 °C.

### Advanced

- Preheater: 5-stage typical, exit gas 300–340 °C, calcination degree at kiln inlet 92–95%.

- Calciner types: in-line, separate-line, downdraft; tertiary air duct; NOx control (staged combustion, SNCR).

- Kiln: L/D ratio ~14–16 for precalciner kiln, 3–4 rpm, slope 3–3.5%, filling degree 10–15%, retention time ~25–30 min.

- Refractory zones, coating formation, kiln rings/build-ups, snowmen in cooler.

- Cooler: grate load t/d/m², cooling air ~2.0–2.2 Nm³/kg clinker, clinker exit ~65 °C + ambient.

- Emissions & environment: ESP vs bag filter (<30 mg/Nm³, often <10), NOx, SOx, CEMS; CPCB/EU norms.

- Alternative fuels & raw materials, co-processing, chlorine bypass.

- Blended cements: OPC/PPC/PSC/composite; BIS IS 269/1489/455; LC3 and CO₂ reduction trends.

### Interview questions

- Explain the cement process end-to-end in 3 minutes. (Practice this out loud — it is asked in almost every interview.)

- What is LSF and what happens if it is too high? Higher free lime, harder burning, higher heat consumption, unstable coating.

- Why a precalciner? Shifts ~60% of fuel to the calciner → smaller kiln, higher output, better fuel efficiency, lower thermal NOx.

- Free lime is high — troubleshoot. Raw mix LSF/fineness, kiln feed variation, burning zone temperature, flame shape, coating, kiln speed/filling, fuel quality/fineness, false air.

- Preheater blockage — causes and prevention. Alkali/sulphur/chloride build-up, high volatiles, cold flushing, poor air cannons; prevent with bypass, air blasters, coating control.

- Difference between ESP and bag filter for kiln exhaust. BF: lower guaranteed emission, unaffected by CO trips; ESP: lower ΔP/power but shuts on CO trip and sensitive to dust resistivity. Modern plants use bag filters.

- What are typical guarantee figures you quoted in tenders? Capacity TPD, kcal/kg clinker, kWh/t cement, emissions mg/Nm³, availability %, noise, product fineness.

# TOPIC 7 — PFD (PROCESS FLOW DIAGRAM)

### Basics

- Purpose: overall process concept, major equipment, main streams, mass/energy data.

- Contents: equipment with tags & names, main process lines with direction, stream numbers + a heat & material balance table (flow, T, P, composition, phase), major control loops only, battery limits.

- Not shown: minor piping, all valves, instrument details, pipe class, spares.

### Advanced

- Development sequence: BFD → PFD → UFD (utility flow diagram) → P&ID.

- Design basis link: capacity, turndown, design margins (usually 10–20%).

- Revision control, hold list, IFR/IFA/IFC status (Issued For Review/Approval/Construction).

- Multiple cases: normal / max / min / startup / upset.

### Interview questions

- PFD vs P&ID vs BFD — difference?

- What information appears in the stream table and why does the client need it?

- Who uses the PFD downstream? Equipment sizing, P&ID development, plot plan, utilities summary, cost estimation, vendors.

- What is battery limit and why is it critical in an EPC contract? Defines scope/responsibility split and tie-in conditions (T, P, flow) — the main source of claims if vague.

# TOPIC 8 — P&ID (PIPING & INSTRUMENTATION DIAGRAM)

### Basics

- Shows every line, valve, instrument, equipment, insulation, tag, line number, and the control loop philosophy.

- Line number format: size – service/fluid code – sequence no. – pipe class – insulation code (e.g. 6"-CW-1203-A1A-IH).

- ISA/ISO 10628 & ISA 5.1 instrument tags: first letter = measured variable (F, P, T, L, A, S), succeeding letters = function (I indicate, C control, T transmit, R record, A alarm, S switch, V valve). Example FIC-101, PSV-201, LAHH-305.

- Circle symbols: plain = field mounted, single line = DCS/control room, dashed = shared display; hexagon = computer function; square-circle = PLC.

- Valve types & symbols: gate, globe, ball, butterfly, check, plug, diaphragm; actuator types; fail position (FC/FO/FL).

- Signal lines: pneumatic, electric, capillary, software link.

### Advanced

- Control loops: flow control, level control, cascade, ratio, split-range, override.

- Fail-safe philosophy and how it is decided (process safety, not preference).

- Safety: PSV/relief sizing basis, rupture disc, flare/vent routing, blowdown, car-sealed open/closed valves.

- Interlocks & cause–effect matrix, SIL basics (IEC 61511), ESD levels.

- Utility stations, drains, vents, sample points, spectacle blinds, minimum flow recycle for pumps, strainers, isolation philosophy for maintenance (double block & bleed).

- P&ID review cycles: 30/60/90% model reviews, HAZOP-mark-up, as-built.

### Interview questions

- Explain a level control loop on a tank from the P&ID. LT → LIC → LCV on outlet; specify fail position and high/low alarms and HH interlock.

- What does `FV-101 FC` mean? Flow control valve 101, fails closed on air/signal failure.

- Why do pumps have a minimum-flow recirculation line? To avoid overheating and low-flow damage during throttled operation.

- How is a control valve sized? Cv calculation at min/normal/max flow, 20–80% opening at normal, check for cavitation/flashing and noise.

- Where do you place a PSV and what sets its size? Governing scenario — blocked outlet, fire case, thermal expansion, control valve failure open.

- What changes on the P&ID after a HAZOP? Added alarms, interlocks, relief devices, isolation, revised fail positions.

# TOPIC 9 — EQUIPMENT SIZING & FUNDAMENTALS

### Pumps

- P (kW) = ρ·g·Q·H / (1000·η), or = Q(m³/h)·H(m)·SG / (367·η). Motor margin 10–20%.

- Types: centrifugal, positive displacement, screw, slurry; selection by viscosity/solids/head.

- Datasheet items: fluid, SG, viscosity, temperature, capacity (rated/normal), differential head, NPSHa, seal plan (API 682), material, driver.

### Fans

- P (kW) = Q(m³/s)·Δp(Pa) / (1000·η). Fan efficiency 70–85%.

- Types: centrifugal (radial, backward-curved, forward-curved), axial; dust-laden gas → radial tip.

- Key cement fans: ID fan, raw mill fan, cooler vent fan, bag filter fan, cooling fans. Correct to actual density at temperature.

- Fan laws, system resistance, damper vs IGV vs VFD.

### Heat exchangers

- Duty → LMTD → U estimate → area → tube count/length → pressure drop check.

### Tanks / silos / bins

- Volume = flow × residence time / fill factor; working vs total volume; silo capacity days-of-storage (clinker 7–15 days, cement 3–7 days typical).

### Cyclones / separators / dust collection

- Cyclone efficiency vs cut size, ΔP 80–150 mmWG per stage; preheater stages total ΔP 550–700 mmWG.

- Bag filter: air-to-cloth ratio (0.9–1.2 m/min pulse jet), can velocity, bag material (polyester/aramid/PPS/PTFE), cleaning system, ΔP 100–150 mmWG.

- ESP: SCA (specific collection area), migration velocity, fields.

### Conveying

- Belt conveyor capacity & power (CEMA), belt width vs lump size, incline limits (~18°).

- Bucket elevator, screw, drag chain, air slide, pneumatic conveying.

### Compressors & utilities

- Compressed air (6–7 barg), instrument air quality, cooling water, plant water, DG backup.

### Interview questions

- Calculate the motor rating for a pump: 200 m³/h, 45 m head, SG 1.0, η 72%. 200×45×1/(367×0.72) ≈ 34 kW → select 37/45 kW motor with margin.

- Why size a fan on actual (Am³/h) and not normal (Nm³/h) volume? Density falls with temperature; a fan at 350 °C handles ~2× the actual volume of the same mass flow at NTP.

- A fan draws more power than designed — causes? Higher gas density (lower temp), higher flow, false air, damper position, wrong impeller, ducting resistance change.

- How do you select the air-to-cloth ratio for a bag filter? Dust type/load, temperature, moisture, cleaning system, emission guarantee; lower ratio for fine/sticky dust.

- What margins do you apply during proposal sizing and why? Typically 10% flow / 5–10% head; too much margin = uncompetitive price and off-BEP operation.

# TOPIC 10 — PROCESS DESIGN BASIS & EPC DOCUMENTATION

### Basics — the core deliverable set

- Design basis / basic engineering package (BEP)

- Process description

- PFD + heat & material balance

- P&ID

- Equipment list

- Process datasheets (equipment + instruments)

- Line list, valve list, instrument list

- Utility summary & utility balance

- Control & operating philosophy, cause–effect matrix

- Technical specifications

- Plot plan / layout inputs

- HAZOP report, safety documents

- Vendor drawing review records

- Operating manual & commissioning support docs

### Advanced

- Design basis contents: capacity & turndown, feed characteristics, product specs, site data (altitude, ambient, seismic, wind), utilities available, codes & standards, battery limits, design margins, sparing philosophy, environmental limits, guarantees.

- Document numbering, revision & transmittal, MDR (Master Document Register), squad check, IDC (inter-discipline check), IFC issue.

- Interfaces: Process ↔ Mechanical (datasheets), ↔ Piping (line list, stress), ↔ Electrical (load list), ↔ Instrumentation (I/O list), ↔ Civil (loads, foundations), ↔ Procurement (RFQ, TBE), ↔ Construction/Commissioning.

- Codes: IS, ASME B31.3/B31.1, API 610/611/650/682, TEMA, IEC, ISA, NFPA, ATEX; local statutory (Factories Act, CPCB, PESO).

### Interview questions

- What goes into a process design basis and who approves it?

- What is IDC/squad check? Cross-discipline check before issue; prevents rework and change orders.

- How do you handle a client change after IFC drawings? Impact assessment → change order/variation → schedule & cost impact → formal approval.

- What is a hold and how is it managed? Undefined data marked on drawings, tracked in a hold register, cleared before IFC.

# TOPIC 11 — PROPOSAL / TENDERING / EPC COMMERCIAL (YOUR STRENGTH — SHOW DEPTH)

### Basics

- Tender lifecycle: RFQ/ITB → bid/no-bid → tender study → technical solution → vendor RFQ → costing → risk review → proposal → clarifications → technical bid evaluation (TBE) → negotiation → LOI/PO → handover to execution.

- Documents: scope of supply, scope of work, exclusions, technical deviation list, commercial deviation list, guarantees, terms of payment, delivery schedule, price schedule/BOQ.

- Contract types: LSTK/EPC lumpsum, EPCM, item-rate, cost-plus, unit rate, BOOT.

### Advanced

- CAPEX estimation: factored estimates (Lang factor, Hand factors), equipment cost + installation factor; Class 5→Class 1 AACE estimate classes with accuracy ±50% → ±10%; escalation, contingency, currency, FOREX, freight, duties, taxes (GST/customs), insurance, site establishment, P&G.

- Cost curves & six-tenths rule: C2 = C1 (Q2/Q1)^0.6.

- OPEX: power, fuel, refractory, grinding media, spares, manpower, maintenance %, consumables → cost/t cement.

- Economics: NPV, IRR, payback, LCOE-style cost per tonne; sensitivity.

- Risk: liquidated damages (LD) for delay & performance shortfall, bank guarantees (ABG/PBG), warranty/DLP, force majeure, price validity, escalation clause, back-to-back with vendors, consortium/JV.

- Technical bid evaluation: normalise scope, evaluate on total evaluated cost = price + power/fuel capitalisation + spares + deviations; technical compliance matrix; vendor qualification/PQ criteria.

- Winning strategy: value engineering, optimum plant configuration, local vs imported equipment, lifecycle cost argument.

### Interview questions

- Walk me through how you built a tender from RFQ to submission. (Use a real cement project story — STAR format.)

- How do you evaluate two vendors when one is cheaper but consumes more power? Capitalise power over the guarantee/evaluation period (e.g., ₹/kWh × kWh/t × capacity × years) and add to bid price; also weigh reliability and spares.

- How do you handle a scope gap discovered after bid submission? Raise a clarification/deviation; if post-award, change order with cost & time impact.

- What is the difference between LSTK and EPCM and which risks shift? LSTK: contractor takes cost/schedule/performance risk, higher price; EPCM: owner keeps risk, contractor provides services.

- How do you estimate CAPEX for a 3000 TPD greenfield line quickly? Benchmark ₹/TPD or $/t capacity from past projects, adjust for scope, location, and escalation; then refine with equipment-wise budgetary quotes.

- What are typical exclusions you always list? Land, statutory approvals, power at battery limit, water source, taxes/duties beyond scope, geotechnical unknowns, client-supplied items, site infrastructure.

- How do you protect against LD exposure at bidding stage? Realistic schedule, cap LD %, back-to-back vendor LDs, milestone-linked guarantees, clear performance test conditions.

- What is a performance guarantee test (PGT)? Defined-duration test with agreed feed/fuel/product conditions to demonstrate capacity, consumption, emissions; define tolerances and repeat/rectification rights.

# TOPIC 12 — CONTROL & OPERATING PHILOSOPHY

### Basics

- Control loop: sensor → transmitter → controller (PID) → final element.

- P, I, D actions; direct/reverse action; open vs closed loop; feedback vs feedforward.

- Instruments: orifice/venturi/mag/ultrasonic flow, RTD/thermocouple, DP/radar/level, pressure transmitter, gas analyser, weigh feeder, belt weigher.

### Advanced

- Operating philosophy: startup, normal, shutdown, emergency, interlock hierarchy, permissives, local/remote, group start sequences.

- Cause & effect matrix and ESD levels; typical cement interlocks (conveyor sequence start against material flow, ZSS/pull-cord/belt-sway, bag filter fan-before-feed, kiln drive backup on DG).

- DCS/PLC/SCADA architecture, I/O list, redundancy, MCC/PCC interface.

- Advanced: expert kiln control, model predictive control, cross-belt analysers, mill optimiser.

### Interview questions

- Explain a kiln control philosophy in simple terms. Control burning zone temperature and free lime via fuel, kiln speed and feed; maintain kiln inlet O₂ and preheater draft with ID fan.

- Why do conveyors start in reverse direction of material flow? To avoid material pile-up on a stopped downstream unit.

- PID tuning — what happens if integral is too aggressive? Oscillation/overshoot and instability.

- What is a permissive vs interlock? Permissive allows start; interlock trips/blocks during operation.

# TOPIC 13 — HAZOP & PROCESS SAFETY

### Basics

- Node → Parameter → Guideword → Deviation → Cause → Consequence → Safeguard → Recommendation.

- Guidewords: No, More, Less, Reverse, As well as, Part of, Other than.

- Parameters: flow, pressure, temperature, level, composition, reaction.

- Team: chairman/facilitator, scribe, process, mechanical, instrumentation, operations, safety.

### Advanced

- HAZID, What-if, FMEA, LOPA, SIL determination & verification, risk matrix (likelihood × severity), ALARP.

- Independent protection layers, safety instrumented functions, PFD (probability of failure on demand).

- Cement-specific hazards: hot meal flush, CO/explosion in coal mill & bag filter (inertisation, CO monitoring, N₂/CO₂ system), dust explosion pentagon, confined space, silo entry, high-temperature surfaces, refractory work, kiln entry.

- Relief & venting, explosion vents on coal mill/filters, fire fighting.

### Interview questions

- Run a HAZOP node for "no flow at pump discharge". (Give causes, consequences, safeguards — as in the roadmap example.)

- Why is a coal mill circuit particularly hazardous? Fine coal + oxygen + heat → fire/explosion; controls: O₂/CO monitoring, temperature limits, inerting, explosion vents, hot-spot avoidance.

- Difference between HAZOP and LOPA? HAZOP is qualitative deviation-based; LOPA is semi-quantitative to test if protection layers reduce risk to tolerable.

- What is SIL and how is it assigned? Safety Integrity Level 1–4 from risk reduction factor needed; assigned via risk graph or LOPA.

# TOPIC 14 — EXCEL (ENGINEERING PRODUCTIVITY)

### Basics

XLOOKUP/VLOOKUP, INDEX+MATCH, IF/IFS, SUMIFS/COUNTIFS, data validation, conditional formatting, named ranges, absolute references, charts, Pivot Tables, Goal Seek, Solver, What-If/Data Table.

### Advanced

- Engineering calc sheets with unit tracking, input/output colour convention, validation checks, error trapping (IFERROR).

- Costing/BOQ templates, comparative statement (bid tabulation) automation.

- Power Query for merging vendor quotes, Power Pivot, dynamic arrays (FILTER, UNIQUE, SORT, LET).

- Basic VBA macro for repetitive datasheet generation.

- Dashboards: KPI cards, slicers, S-curve for project progress.

### Interview questions

- Build a bid comparison sheet — how would you structure it? Normalised scope rows, vendor columns, deviations flagged, evaluated cost formula, ranking with INDEX/MATCH.

- When do you use Goal Seek vs Solver? Single variable target vs multi-variable constrained optimisation.

- INDEX/MATCH vs VLOOKUP advantage? Left lookups, insertion-safe, faster on large data.

# TOPIC 15 — AUTOCAD (READ-LEVEL COMPETENCE)

### Basics

Interface, units, layers, blocks & attributes, xrefs, dimensioning, text styles, model vs paper space, plotting/scale, DWG/DXF/PDF.

### Advanced

- Title block & revision cloud practice, layer naming standards, block libraries for P&ID symbols.

- Mark-up workflow: red-line → drafter → check → approve; comparing revisions.

- Awareness only: Plant 3D / SmartPlant P&ID / E3D, 3D model reviews, clash detection, isometrics.

### Interview questions

- How do you mark up a P&ID revision? Cloud + revision triangle + revision table entry + transmittal.

- What is an xref and why is it used? Referenced external drawing (e.g., plot plan) to keep single source of truth.

# TOPIC 16 — ASPEN PLUS / HYSYS (TIER 3, AWARENESS + BASICS)

### Basics

Component list, property/thermodynamic package selection (NRTL, Peng–Robinson, SRK; solids for cement), material streams, unit blocks (mixer, heater, flash, pump, compressor, RGibbs/RStoic reactor), running & reading results, sensitivity analysis, design spec.

### Advanced

Recycle & convergence (Wegstein, Broyden), tear streams, flowsheet optimisation, heat integration/pinch analysis, dynamic simulation basics.

For cement: solids modelling is limited — mention Excel-based heat & mass balance is the industry norm; simulation is more relevant if you move to chemical/refinery/EPC consultancy.

### Interview questions

- How do you choose a property package? Based on components and conditions — PR/SRK for hydrocarbons/gases, NRTL/UNIQUAC for polar/non-ideal liquids.

- Simulation doesn't converge — what do you do? Check spec feasibility, initialise/tear streams, relax tolerance, simplify flowsheet, provide better estimates.

# TOPIC 17 — MOCK PROJECT (YOUR INTERVIEW PORTFOLIO)

Build one folder, 14 deliverables, for a simplified line (e.g., 1 MTPA cement grinding unit or 3000 TPD clinker line):

- Process description (2 pages)

- Design basis (capacity, feed analysis, site data, utilities, guarantees)

- PFD with stream table

- Mass balance (Excel)

- Energy balance (Excel, kcal/kg clinker)

- Equipment list with tags and duty

- Sizing calcs: 1 fan, 1 pump, 1 bag filter, 1 silo, 1 conveyor

- Two equipment datasheets

- Utility summary (power, water, compressed air)

- P&ID of one system (e.g., cement mill or packing plant)

- Control philosophy + cause & effect matrix for that system

- HAZOP worksheet — 2 nodes

- Technical specification for one equipment

- CAPEX estimate (equipment-wise + factored) and simple OPEX/t

Interview use: "I built a complete basic engineering package for a 3000 TPD line — here is my heat balance and here is why I selected a 5-stage preheater." That single sentence beats any certificate.

# TOPIC 18 — BEHAVIOURAL & POSITIONING (4-YEAR LEVEL)

### Your positioning statement

"I have ~4 years in cement EPC and tendering — technical specifications, scope definition, vendor evaluation, multidisciplinary coordination on greenfield and brownfield projects, domestic and international. I am now deepening the process engineering side: mass and energy balances, PFD/P&ID, equipment sizing and process documentation, so I can own the technical solution end-to-end, not just compile it."

### Expected behavioural questions

- Tell me about yourself. (90 seconds: background → domain → what you own → why this role.)

- Most complex tender you handled and your specific contribution.

- A time you found an error late — what did you do?

- How do you handle conflicting inputs from mechanical and electrical during a bid?

- A client rejected your technical offer — what happened next?

- How do you prioritise when three tenders are due the same week?

- Why move from tendering to process/EPC engineering?

- Where do you see yourself in 3 years? (Lead process/proposal engineer owning full BEP + guarantees.)

- Salary expectation & notice period.

- Your questions to them: team structure, project pipeline, which stage of engineering they own, tools used, growth path.

### STAR answer skeleton

Situation (project, capacity, client, country) → Task (your responsibility) → Action (calculations, coordination, decisions) → Result (won order value, cost saved, schedule met, deviation closed).

# TOPIC 19 — 16-WEEK STUDY SCHEDULE (2–3 h/day)

| Week | Focus | Deliverable |
| --- | --- | --- |
| 1–2 | Mass balance + units + cement flow chain | 3 solved balances in Excel |
| 3–4 | Energy balance + kiln heat balance + fuels | Full kiln heat balance sheet |
| 5 | Fluid mechanics + pumps + fans sizing | Pump & fan calc sheet |
| 6 | Heat transfer + cyclones + bag filters | Bag filter & cyclone sizing |
| 7–8 | PFD + P&ID symbols, tags, loops | Redraw a PFD & P&ID by hand/CAD |
| 9 | Cement chemistry, moduli, KPIs, benchmarks | One-page KPI cheat sheet |
| 10 | EPC documentation set + design basis | Draft design basis for mock project |
| 11 | Equipment datasheets + technical specs + TBE | 2 datasheets + 1 bid comparison |
| 12 | CAPEX/OPEX, contracts, guarantees, LDs | CAPEX estimate for mock project |
| 13 | Control philosophy + cause & effect | C&E matrix |
| 14 | HAZOP + process safety | 2-node HAZOP worksheet |
| 15 | AutoCAD + Excel advanced | Simple P&ID drawn in CAD |
| 16 | Mock project assembly + interview drills | Portfolio PDF + 20 rehearsed answers |

Apply from Week 6 onward — do not wait for Week 16.

# TOPIC 20 — RAPID-FIRE NUMBERS TO MEMORISE

| Parameter | Typical value |
| --- | --- |
| Heat consumption, 5-stage PH+PC kiln | 690–730 kcal/kg clinker |
| Theoretical heat of clinkerisation | 420–450 kcal/kg |
| Total specific power | 90–110 kWh/t cement |
| Raw meal : clinker factor | ~1.55–1.65 |
| Calcination temperature | ~900 °C |
| Clinkerisation / burning zone | 1350–1450 °C material, ~2000 °C flame |
| Preheater exit gas temp | 300–340 °C |
| Kiln inlet calcination degree | 92–95% |
| Free lime target | 0.8–1.5% |
| LSF / SM / AM | 92–98 / 2.3–2.7 / 1.3–1.6 |
| Cooling air | ~2.0–2.2 Nm³/kg clinker |
| Blaine OPC / PPC | 300–400 / 400–500 m²/kg |
| Bag filter A/C ratio (pulse jet) | 0.9–1.2 m/min |
| Dust emission norm | < 30 mg/Nm³ (often < 10) |
| CO₂ per tonne cement | ~0.55–0.9 t |
| Duct gas velocity | 15–20 m/s |
| Water pipe velocity | 1–3 m/s |
| Pump power formula | Q·H·SG / (367·η) kW |
| Fan power formula | Q(m³/s)·Δp(Pa) / (1000·η) kW |

## Resources

- Books: Duda Cement Data Book; Peray The Rotary Cement Kiln; Bhatty/Miller/Kosmatka Innovations in Portland Cement Manufacturing; Coulson & Richardson Vol. 1 & 6; Perry's Chemical Engineers' Handbook (reference); Ludwig for equipment design.

- Standards: IS 269/455/1489/8112, ASME B31.3, API 610/682/650, TEMA, ISA 5.1, IEC 61511.

- Free: FLSmidth/thyssenkrupp/KHD technical brochures, CII/NCB cement benchmarking reports, Cement Manufacturers' Association energy benchmarking, CemNet training pages.

- Practice: rebuild every calculation in Excel yourself — do not just read.